/*!

\page ghs3d_hypo_page GHS3D Parameters hypothesis

\n GHS3D Parameters hypothesis works only with <b>Tetrahedron (GHS3D)</b>

algorithm. This algorithm is a commercial software.

<h1>General parameters</h1>

\image html ghs3d_parameters_basic.png

<ul>

<li><b>Name</b> - allows to define the name of the hypothesis (GHS3D

Parameters by default).</li>

<li><b>To mesh holes</b> - if checked, the algorithm will

create mesh in the holes inside a solid shape, else only the outermost

shape will be meshed. Volumic elements created within holes are bound

to the solid.</li>

<li><b>Optimization level</b> - allows choosing the required

optimization level: none, light, medium or strong. Higher level of

optimisation provides better mesh, but can be time-consuming.

</li>

<h1>Advanced parameters</h1>

\image html ghs3d_parameters_advanced.png

<li><b>Maximum memory size</b> - launches ghs3d software with

work space limited to the specified amount of RAM, in Mbytes. If this option is

checked off, the software will be launched with 7O% of the total RAM space. </li>

<li><b>Initial memory size</b> - starts ghs3d software with

the specified amount of work space, in Mbytes. If this option is checked off, the

software will be started with 100 Megabytes of working space. </li>

<li><b>Working directory</b> - allows defining the folder for input and output

files of ghs3d software, which are the files starting with "GHS3D_" prefix. </li>

<li><b>Keep working files</b> - allows checking input and output files

of ghs3d software, while usually these files are removed after the

launch of the mesher.</li>

<li><b>Verbose level</b> - to choose verbosity level in the range from

0 to 10.

<ul> <li>0, no standard output,

</li><li>2, prints the data, quality statistics of the skin and final

meshes and indicates when the final mesh is being saved. In addition

the software gives indication regarding the CPU time.

</li><li>10, same as 2 plus the main steps in the computation, quality

statistics histogram of the skin mesh, quality statistics histogram

together with the characteristics of the final mesh.

</li></ul></li>

<li><b>To create new nodes</b> - if this option is checked off, ghs3d

tries to create tetrahedrons using only the nodes of the 2D mesh.</li>

<li><b>To remove the initial central point</b> TetMesh-GHS3D adds an internal point

at the gravity centre of the bounding box to speed up and to simplify

the meshing process. However, it is possible to refrain from creating

this point by using the command line option -no initial central point. This can be

particularly useful to generate a volume mesh without internal points at all and in some rare cases

at the boundary regeneration phase when it is impossible to proceed

with the standard options

(for example, when one dimension of the domain is more than 20 times greater than the other two).

Use this option if the boundary regeneration has failed with the standard parameters and before using

the recovery version (command line option -C).

Note: when using this option, the speed of the meshing process may

decrease, and the quality may change.

Note: the boundary regeneration may fail with this option, in some rare cases.</li>

<li><b>To use boundary recovery version</b> - enables using a

boundary recovery module which tries to

create volume meshes starting from very poor quality surface meshes

(almost flat triangles on the surface, high density propagation,

extreme aspect ratios, etc.) which fails with the standard version. The

resulting volume mesh will however most likely have a very poor

quality (poor aspect ratio of elements, tetrahedra with a very small

positive volume).</li>

<li><b>To use FEM correction</b> - Applies finite-element correction by

replacing overconstrained elements where it is possible. At first the process

slices the overconstrained edges and at second the overconstrained

facets. This ensures that there are no edges with two boundary

vertices and that there are no facets with three boundary vertices. TetMesh-GHS3D gives the initial

and final overconstrained edges and facets. It also gives the facets

which have three edges on the boundary.

Note: when using this option, the speed of the meshing process may

decrease, quality may change, and the smallest volume may be smaller.

By default, the FEM correction is not used.</li>

<li><b>Option as text</b> - allows to input in the command line any text

for ghs3d, for example, advanced options. </li>

</ul>

<h1>Enforced vertices</h1>

\image html ghs3d_enforced_vertices.png

GHS3D algorithm can locally make the mesh finer. It is possible to define enforced vertices in the volume where the mesh will be detailed.

A node will be created at the enforced vertex coordinates. There is no need to create a vertex in CAD.

An enforced vertex is defined by:

<ul>

<li>The (x,y,z) cartesian coordinates</li>

<li>A constant physical size</li>

</ul>

<br><b>See Also</b> a sample TUI Script of the \ref tui_ghs3d "creation of a Ghs3D hypothesis", including enforced vertices.

*/