/*!

\page pattern_mapping_page Pattern mapping

<br><h2>About patterns</h2>

The pattern describes a mesh to generate: positions of nodes within a

geometrical domain and nodal connectivity of elements. As well, a

pattern specifies the so-called key-points, i.e. nodes that will be

located at geometrical vertices. Pattern description is stored in

\<pattern_name\>.smp file.

The smp file contains 4 sections:

<ol>

<li>The first line holds the number of nodes (N).</li>

<li>The next N lines describe nodes coordinates. Each line holds 2

coordinates of a node.</li>

<li>A key-points line: indices of nodes to be mapped on geometrical

vertices. An index n refers to a node described on an n-th line of

section 2. The first node index is zero.</li>

<li>The rest lines describe nodal connectivity of elements, one line

for an element. A line holds indices of nodes forming an element. An

index n refers to a node described on an n-th line of the section

2. The first node index is zero. There must be 3 or 4 indices on a

line: only 2d elements are allowed.</li>

</ol>

The 2D pattern must contain at least one element and at least one

key-point. All key-points must lay on boundaries.

An example of a simple smp file and a preview of a pattern described

in this file:

\image html image94.gif

<br><h2>Application of pattern mapping</h2>

<em>To apply pattern mapping to a geometrical object:</em>

From the \b Modification menu choose the <b>Pattern Mapping</b> item or click

<em>"Pattern mapping"</em> button in the toolbar.

\image html image98.png

<center><em>"Pattern mapping" button</em></center>

The following dialog box shall appear:

\image html patternmapping1.png

\image html patternmapping2.png

To apply a pattern to a geometrical object, you should specify:

<ul>

<li>a face having the number of vertices equal to the number of

key-points in the pattern; the number of key-points on internal

boundaries of a pattern must also be equal to the number of vertices

on internal boundaries of a face;</li>

<li>a vertex to which the first key-point should be mapped;</li>

<li>reverse or not the order of key-points. (The order of vertices of

a face is counterclockwise looking from outside).</li>

</ul>

Then you either load a .smp pattern file previously created manually

by clicking on the <em>"Load pattern"</em> button, or click on the \b

New button for automatic generation.

\n For an automatic generation you just specify a geometrical face

having a mesh built on it. Mesh nodes lying on face vertices become

key-points. Additionally, you may choose the way of getting nodes

coordinates by <b>projecting nodes on the face</b> instead of using

"positions on face" generated by mesher (if there is any). Faces

having a seam edge can?t be used for automatic pattern creation.

When creating a pattern from an existing mesh, there are two possible

cases:

<ol>

<li>A sub-mesh on face is selected. A pattern is created from the 2d

elements bound to a face by mesher. Node coordinates are either

"positions on face" computed by mesher, or coordinates got by node

projection on a geometrical surface, according to your choice.</li>

<li>A mesh where the main shape is a face, is selected. A pattern is

created from all the 2d elements in a mesh. If all mesh elements are

build by mesher, the user can select the way of getting nodes

coordinates, else all nodes are projected on a face surface.</li>

</ol>

\image html a-patterntype.png

\image html a-patterntype1.png

<br><h2>Mapping algorithm</h2>

The mapping algorithm is as follows:

<ol>

<li>Key-points are set in the order that they are encountered when

walking along a pattern boundary so that elements are on the left. The

first key-point is preserved.

</li>

<li>Find geometrical vertices corresponding to key-points by vertices

order in a face boundary; here, "Reverse order of key-points" flag is

taken into account.

\image html image95.gif

</li>

<li>Boundary nodes of a pattern are mapped onto edges of a face: a

node located between certain key-points on a pattern boundary is

mapped on a geometrical edge limited by corresponding geometrical

vertices. Node position on an edge reflects its distance from two

key-points.

\image html image96.gif

</li>

<li>Coordinates of a non-boundary node in a parametric space of a face

are defined as following. In a parametric space of a pattern, a node

lays at the intersection of two iso-lines, each of which intersects a

pattern boundary at least at two points. Knowing mapped positions of

boundary nodes, we find where isoline-boundary intersection points are

mapped to, and hence we can find mapped isolines direction and then,

two node positions on two mapped isolines. The eventual mapped

position of a node is found as an average of positions on mapped

isolines.

\image html image97.gif

</li>

</ol>

<br><b>See Also</b> a sample TUI Script of a

\ref tui_pattern_mapping "Pattern Mapping" operation.

*/